LOCOMOTIVE SHED, SABARMATI

ELECTRICAL

[3-फेज विद्युत लोकोमोटिव आईए/आईबी/आईसी अनुसूची [निरीक्षण]

IA/IB/IC Schedule of 3-Phase Electric Locomotive [Inspection]

लोको संख्या Loco No. 	                    शिड्यूल Sch.                     शिड्यूल में लेने का दिनांक Date taken under Sch.

| AUXILIARY MACHINES SCHEDULE |  |  |  |  |
| --- | --- | --- | --- | --- |
| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) ###### Detail of Work/ Inspection (schedule) | की गयी कार्यवाही Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर/ टिप्पणी Sign/ Remarks |
| A | ### Oil circulating pumps (SR & TFP) |  |  |  |
|  | 1) Visually examine all SR&TFP oil pumps (Four nos.) for any oil leakage / any abnormal sound and take needful action. |  |  |  |
|  | 2) Check the mechanical support fasteners of all four oil pumps. |  |  |  |
|  | 3) Check and secure rubbing of SR/TFP Cable. |  |  |  |
| B | During schedule check all Auxiliary Blower and Motors: - |  |  |  |
|  | 1) Check tightness of foundation bolts of all auxiliary motors. |  |  |  |
|  | 2) Visual examination / RDPT of impeller of OCB. |  |  |  |
|  | 3) Check the condition of all flexible hoses connected with various auxiliary blowers and replace, if required. |  |  |  |
|  | 4) Check tightness of SB and cable connections at terminal box. |  |  |  |
|  | 5) Ensure that the terminal covers of various auxiliary motors are intact and water tight. Special attention needs to be given to main compressor, OCB, TMB motor cable layout and water tightness of its entry point into the motor terminal. |  |  |  |
|  | 6) Clean commutator of CPA motor. Check carbon brush and its holding assembly and replace worn out / defective carbon brushes and associated equipments. |  |  |  |
|  | 7) Check availability of earthing shunt. |  |  |  |
|  | 8) Check condition of cab fan foundation, secure rubbing of wire, and check freeness of fan blade. |  |  |  |
| C | IC Sch.: Besides of the above Sch. following attention needs to be given: - |  |  |  |
|  | 1) Check the electrical connections of all four oil pumps. |  |  |  |
|  | 2) Check the electrical connections of all auxiliary motors. |  |  |  |
|  | 3) Greasing of all motors with recommended grease or RR-3 Servo Gem Grease ,6 to 7 Stroke ,15 gm. |  |  |  |

Remarks if any:

पर्यवेक्षक के हस्ताक्षर Signature of JE/SSE