LOCOMOTIVE SHED, SABARMATI

ELECTRICAL

[3-फेज विद्युत लोकोमोटिव आईए/आईबी/आईसी अनुसूची [निरीक्षण]

IA/IB/IC Schedule of 3-Phase Electric Locomotive [Inspection]

लोको संख्या Loco No. 	                    शिड्यूल Sch.                     शिड्यूल में लेने का दिनांक Date taken under Sch.

| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) Detail of Work/ Inspection (Schedule) | कार्रवाई की गयी Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर / टिप्पणी Sign / Remarks |
| --- | --- | --- | --- | --- |
| A | # ड्राइविंग डेस्क (Driving Desk) |  |  |  |
| 1 | Clean the surfaces of the driver’s desk with a soap solution and cloth, taking care not to allow water to enter electrical cubicles. |  |  |  |
| 2 | Clean the dust filter on the buzzer using a soft brush without using the pressure / compressed air. |  |  |  |
| 3 | Ensure that transparent rubber caps provided over the push buttons of driver desk. |  |  |  |
| 4 | Clean the battery, OHE and TE/BE meters in both cabs and check for zero error |  |  |  |
| 5 | Check tightness/operation/cable rubbing of connector, earthing of angle transmitter, all rubber gasket and all equipment located on BL and its box frame inside the panel. Clean contacts, check cap and its fitment tightness. |  |  |  |
| 6 | Check tightness of sub D located on BL. |  |  |  |
| 7 | Check ZPT switch located on pneumatic panel. |  |  |  |
| 8 | Check all earthing points located on equipment. |  |  |  |
| B | LIGHTING AND AIRFLOW |  |  |  |
|  | हेड लाइट, मार्कर लाइट,फ्लेशरलाइट,कैब व कोरिडोर लाइट HEAD LIGHT,CAB & CORIDOOR LIGHT, FLASHER LIGHT,MARKER LIGHT |  |  |  |
| 1 | Check lighting units for physical damage, tightness of fixing connections, particularly look for signs of overheating or burning. Replace bulbs/tubes, which have failed. |  |  |  |
| 2 | Clean all internal light unit diffusers, bulbs or tubes and reflectors. |  |  |  |
| 3 | Clean the glasses of the headlight lamp and marker light. |  |  |  |
| 4 | Ensure that there is no entry of water through head light lamps. |  |  |  |
| 5 | Check for crack or breakage of the aluminum casing of headlight beams. |  |  |  |
| 6 | Check the proper operation of flasher light, marker light (red & white), headlight, cab lights, disc light, m/c. room light. |  |  |  |
| 7 | Check headlight holder and clean glass & reflector. check tightness of headlight convertor, Ensure sealing . |  |  |  |
| 8 | Clean marker light glass and check tightness of connection. Check looseness, flashing in flasher unit. |  |  |  |

| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) Detail of Work/ Inspection (Schedule) | कार्रवाई की गयी Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर / टिप्पणी Sign / Remarks |
| --- | --- | --- | --- | --- |
| C | SB1 & 2, HB1 & 2 AND FB PANEL / CUBICLE |  |  |  |
| 1 | Check the mounting block of 8.1 and 8.2 contactors by opening side windows and check the main contactor, pre charging contactor foundation bolt tips for looseness and overheating by removing arc-chutes from front side. |  |  |  |
| 2 | Check the condition of capacitors for leakage and check earth fault relay, resistance and discharging resistance for overheating. Check the tightness of all fastener lug, LEM sensor and CTs. |  |  |  |
| 3 | Ensure that the various foundations bolts, body support bolts and supports plates of FB cubicle are intact and tight. Check tightness of cubical doors safety locks. |  |  |  |
| 4 | Ensure tightness of various electrical connections, CBS in SB, HB cubicles. |  |  |  |
| 5 | Check back side of HB cubicle all SB connections for tightness/overheating/cable rubbing. |  |  |  |
| 6 | IC Sch.: a) Clean various equipment fitted inside the SB, HB & FB cubicle. |  |  |  |
|  | b) Visual examination of various EPCs & EMCs fitted inside FB cubicle and ensure that there is no crackness of cover cylinder, leakage and they are functioning properly. |  |  |  |
|  | c) Check the tightness of various PCB cards in central electronics. |  |  |  |
|  | d) Check the RC input filter circuit for any abnormality and measure the resistance value. |  |  |  |
|  | e) Take out the arc-chutes of pre-charging contactors for looseness of strips and ensure tightness of its armature locking nut by hand. |  |  |  |
|  | f) Check the FB cable junction for tightness, cable rubbing and overheating. |  |  |  |
| D | HB CUBICLE : |  |  |  |
| 1 | Check all equipments’ fitment, tightness, intactness of base foundation bolts & body support. |  |  |  |
| 2 | Ensure earthing point/shunts of all panels/cubicles. |  |  |  |
| 3 | Check all MCB cable connections & Siemen connector tightness and no overheating. |  |  |  |
| 4 | Check tightness of 80 A & 150 A contactors connection and condition of tips. |  |  |  |
| 5 | Check 1000 V/415 V/110 V transformer connections for overheating and tightness of foundation bolts and checking of fuses for healthy connection. |  |  |  |
| 6 | Check tightness and condition of capacitor, LEM sensor below HB panel, earth fault relay & resistance connection, RLC plate, temperature sensor and all diode boxes. |  |  |  |
| 7 | Checking of OCB cable in HB 1& 2 for overheating cable no.1121, 1122, 1123 (A & B). |  |  |  |

| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) Detail of Work/ Inspection (Schedule) | कार्रवाई की गयी Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर / टिप्पणी Sign / Remarks |
| --- | --- | --- | --- | --- |
| E | SB CUBICLE: |  |  |  |
| 1 | Check fasten lug connected on various equipment. Fitment, tightness. |  |  |  |
| 2 | Check intactness of base foundation bolts & body support. Check tightness of cubical doors safety locks. |  |  |  |
| 3 | Check all EMC connector lug position/layout. It should not be touch with plunger. |  |  |  |
| 4 | Visually check the connections on backside of SB 1 & 2 panel for overheating of WAGO connector etc. |  |  |  |
| 5 | Check tightness of DC-DC converter 24V/48V and Siemen connector for overheating. |  |  |  |
| 6 | Visually check all pressure switches, tightness of MCB, MCR relay, no voltage relay connections. |  |  |  |
| 7 | Key interlocking: Check all keys are present & the complete system is operational. |  |  |  |
| 8 | Ensure the healthiness; Visually check all pre/proper functioning of rotating switch 152, 154 and 237.1 on its various locations. |  |  |  |
| 9 | Check visually for any abnormality in SB, HB & FB cubicle |  |  |  |
| 10 | Ensure proper functioning of thermostat of central electronics. (IC Sch.) |  |  |  |
| F | MAIN ASSEMBLY BOX-1 & BOX-2: |  |  |  |
| 1 | Clean the dust of enclosure using vacuum cleaner. |  |  |  |
| 2 | Check the metallic strip provided for spring action in the contact is intact. Replace the contact if required. |  |  |  |
| 3 | Check the connections for 25, 50 & 70 Sq. mm cables for their tightness. (IC Sch.) |  |  |  |
| 4 | Tightness of knife contact connection (female), 3-phase choke connection, Power contactor connection. (IC Sch.) |  |  |  |

| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) Detail of Work/ Inspection (Schedule) | कार्रवाई की गयी Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर / टिप्पणी Sign / Remarks |
| --- | --- | --- | --- | --- |
| G | CENTRAL ELECTRONIC RACK |  |  |  |
| 1 | Check visually and manually all cards tightness, locking, all sub-D connection tightness, all optic fibre cable tightness for proper layout. Check tightness of cubical doors safety locks. |  |  |  |
| 2 | Check the operation of all fans for free movements. |  |  |  |
| 3 | Check the tightness of connections of control cards. (IC Sch.) |  |  |  |
| H | FIRE DETECTION UNIT |  |  |  |
| 1 | Calibrate the FDU and set voltage in between: - Trolex make: 0 to + 100 mV - Siemens/Cerberus make: 0 to + 50 mV |  |  |  |
| 2 | Check and clean FDU pipe line by vacuum cleaner. (IC Sch.) |  |  |  |
| 3 | Ensure the intactness / firmness of cable connection of Sicemcouplers of differential amplifiers provided below SR oil pump. (IC Sch.) |  |  |  |

Remarks if any:

पर्यवेक्षक के हस्ताक्षर Signature of JE/SSE