LOCOMOTIVE SHED, SABARMATI

ELECTRICAL

[3-फेज विद्युत लोकोमोटिव एच एल सी अनुसूची [निरीक्षण]

HLC Schedule of 3-Phase Electric Locomotive [Inspection]

लोको संख्या Loco No. 	                                    शिड्यूल Sch.                                                           दिनांक  Date

# Schedule for maintenance and activities for Hotel Load Converter:

Activities in IA/IB/IC schedules:

| Sr. | Activities | IA/IB | IC | Deficiency Noticed | Action Taken |
| --- | --- | --- | --- | --- | --- |
|  | Check functioning of the blowers and churning fans. Replace in case of any abnormality. Check direction if required. | √ | √ |  |  |
|  | Inspect the contact surfaces of main/pre- charge/auxiliary contactors for any sign of wear, pitting or erosion. If the contacts are significantly worn, consider replacing them. | √ | √ |  |  |
|  | Clean all Suction Filters of hotel load converter. | √ | √ |  |  |
|  | After schedule of inspection, erase FDP memory. | √ | √ |  |  |
|  | Check parameters of HLC in working conditions. - Output voltage: 750V +5%/-2%; - Line to Earth voltage peak : <800V; - O/p Voltage THD: < 10%; - I/p current THD (full load): <5% (up to 20th harmonics); - O/p voltage unbalance: >3%. | √ | √ |  |  |
|  | Check the sealing of the cable entry into the panel (to stop the dust entry). | √ | √ |  |  |
|  | Check the earth connections of the HLC and Output contactor panels. | √ | √ |  |  |
|  | Check the tightness/intactness of connectors of the panel. | √ | √ |  |  |
|  | Check the door locks of the panel. | √ | √ |  |  |
|  | Check the working of display unit of HLCs & functioning of keypad keys. | √ | √ |  |  |
|  | Visual checking of blower transformer, if available. | √ | √ |  |  |
|  | Cleaning of front door filters, if provided. | √ | √ |  |  |
|  | Check both HLCs on load arrangement for ensuring proper working on line. Run HLC on load for 5-10 minutes. Note current and voltage. Check all indication at control box is proper or not. | √ | √ |  |  |
|  | Check the condition of blocking diode fitted in SB-2 panel as per RDSO MS-488. | √ | √ |  |  |
|  | Ensure proper working of HOG ON Command Indication lamp provided in D- Panel as per RDSO MS-489. | √ | √ |  |  |
|  | Check working of fault reset command with the help of control box. | √ | √ |  |  |
|  | Check air flow at suction filter and exhaust louvers position for ensuring proper working of blower fans. Average suction air flow should be >2.5 m/s for Medha & BHEL make and >3.5 m/s for Siemens & AAL make HLCs. The measurement should be taken at the points mentioned in Figure-1 below: | √ | √ |  |  |

Additional activity in IC schedule

| Sr. | Activities | IA/IB | IC | Deficiency Noticed | Action Taken |
| --- | --- | --- | --- | --- | --- |
|  | Check door gaskets for damages. If found damaged, change the same. | X | √ |  |  |
|  | Check the dust ingress into the HLC panel and output contactor panel. Use a vacuum cleaner to clean. | X | √ |  |  |
|  | Check the cable tie wraps of the power cables. Check for sign of overheating. | X | √ |  |  |
|  | Measure the overall capacitance of each DC link and replace if needed. (12mF ± 5% – Siemens & AAL, 10.8 mF ±5% – Medha, 10.4mF ± 5% – BHEL). Ensure proper tightening torque of fasteners. | X | √ |  |  |
|  | Check tightness of connections at input, output and control cables. | X | √ |  |  |
|  | Check the power connection for color changes/overheating and change the cables and sockets, if required. | X | √ |  |  |
|  | Check tightness of connections of power cables. | X | √ |  |  |
|  | Check the pre-charging contactors for any signs of physical damage, overheating or discoloration. Check for loose connections and ensure that all parts are in good condition. | X | √ |  |  |
|  | Check the contacts of the main contactors for any signs of physical damage, overheating or discoloration. Check for loose connections and ensure that all parts are in good condition. | X | √ |  |  |
|  | Check the contacts of the auxiliary Contactors for any signs of physical damage, overheating or discoloration. Check for loose connections and ensure that all parts are in good condition. | X | √ |  |  |
|  | Clean the heat sinks of HLCs. | X | √ |  |  |
|  | Check the tightness of mounting / fixation bolts with the loco body. | X | √ |  |  |
|  | Check the output contactors and o/p Power cable connection for any overheating physical damage or discoloration. Check for loose connections and ensure that all parts are in good condition | X | √ |  |  |
|  | Visually check the sine filter capacitor and measure its value. Ensure proper tightening torque of fasteners. | X | √ |  |  |
|  | Visually inspect the RC filter and discharge resistors for signs of overheating. | X | √ |  |  |
|  | Visually inspect pre-charge resistor. Replace if found damaged or overheated. | X | √ |  |  |
|  | Check phase sequence of all 04 IV couplers. | X | √ |  |  |

# Figure – 1: Air flow measurement from HLC Suction Filters (Loco side wall)

- Fault log and driver Bookings

- Maintenance of IV Couplers

| Sr. | Activities | IA/IB | IC | Deficiency Noticed | Action Taken |
| --- | --- | --- | --- | --- | --- |
|  | Check IV coupler cover/body for any damage and replace if required. | √ | √ |  |  |
|  | Check visually for any sign of damage, flash mark and overheating. Repair if required. | √ | √ |  |  |
|  | Check for intactness of sealing ring gasket on socket of locomotive. | √ | √ |  |  |
|  | Clean IV coupler and lubricate moving parts if found sluggish. | √ | √ |  |  |
|  | Check IV coupler Ratchet assembly and ensure proper locking/unlocking & ensure intactness of all Hardware (nuts/bolts/washers etc.). | √ | √ |  |  |
|  | Ensure intactness of both HEX small bolt (bearing screw) and both HEX big bolt (Bearing Pivot Bolt). | √ | √ |  |  |
|  | Ensure proper working of locking bolt. | √ | √ |  |  |
|  | Ensure intactness of both Allen screw. | √ | √ |  |  |
|  | Ensure intactness of insulated base foundation bolt. It should be straight to the holes given on the coupler edge. | √ | √ |  |  |
|  | Ensure the proper condition of Lever tie bow Assembly gap. Check with “NO GO Gauge”. | √ | √ |  |  |
|  | Check IV coupler mobile contact pin (Phase & control) for flashing/free movement by pressing and replace if stuck up / sluggish. | √ | √ |  |  |
|  | Clean coupler pins with suitable cleaners. | √ | √ |  |  |
|  | Ensure that couplers are in closed conditions to prevent cover damage and ingress of water & dust. | √ | √ |  |  |
|  | Check phase sequence at all 4 couplers. | √ | √ |  |  |
|  | Check Insulation between Control Pins (C1 & C2) w.r.t. body of IVC at 500 VDC. This should be >10 M ohm | √ | √ |  |  |
|  | Check Insulation between high voltage supply pins w.r.t. body of IVC with 1500 VDC Megger. This should be >100 M ohm. | √ | √ |  |  |

Additional activity in IC schedule

| Sr. | Activities | IA/IB | IC | Deficiency Noticed | Action Taken |
| --- | --- | --- | --- | --- | --- |
| 17 | Replace O ring (sealing gasket) of IV coupler. | X | √ |  |  |

# Maintenance of UIC Couplers

| Sr. | Activities | IA/IB/IC | Deficiency Noticed | Action Taken |
| --- | --- | --- | --- | --- |
| 1. | Both sides UIC coupler cover tension to be checked. If found not proper then replace the same. | √ |  |  |
| 2. | Ensure proper unlocking/locking of covers of both sides UIC couplers before and after inspection. All nut-bolts should be intact. | √ |  |  |
| 3. | Both sides UIC coupler female contact pin to be checked for oxidation/flashing, pin diameter and locking (to be done without opening of UIC coupler). | √ |  |  |
| 4. | UIC coupler must be in closed condition to preventing ingress of dust and water. | √ |  |  |
| 5. | In UIC coupler, condition of all red colour gaskets needs to be checked. | √ |  |  |
| 6. | During load / no load testing of HLCs, functionality of all concerned UIC pins needs to be checked. | √ |  |  |

Any other remarks:

Name of TCN:								Name of Supervisor:

Sign of TCN:								Sign of supervisor: