LOCOMOTIVE SHED, SABARMATI

ELECTRICAL

[3-फेज विद्युत लोकोमोटिव आईए/आईबी/आईसी अनुसूची [निरीक्षण]

IA/IB/IC Schedule of 3-Phase Electric Locomotive [Inspection]

लोको संख्या Loco No. 	                    शिड्यूल Sch.                     शिड्यूल में लेने का दिनांक Date taken under Sch.

| A | # PROPULSION SYSTEM: TRACTION CONVERTER |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | Check the cable connection of Sichem connectors of differential amplifier provided below SR oil pump |  |  |  |
| 2 | Check the condition of silica gel. If pink replace with dried silica gel |  |  |  |
| 3 | IC Sch: |  |  |  |
|  | - Clean resonance circuit capacitor. |  |  |  |
|  | - Cleaning by Vacuum cleaner |  |  |  |
|  | - Clean DC link capacitors by blowing after opening side cover. Check that there is no sign of leakage from the capacitors |  |  |  |
|  | - Clean the isolating blade and spring contact of earthing switch. Lightly lubricate with the specified grease |  |  |  |
|  | - Check the tightness of SR electronics PCB |  |  |  |
|  | - Ensure sealing of incoming cable gland of control card rack to avoid dust entry |  |  |  |
|  | - Check the ventilation fans of traction converters |  |  |  |
| B | # AUXILIARY CONVERTER 1, 2 & 3 |  |  |  |
| 1 | Visually check the connection of choke and transformer for overheating |  |  |  |
| 2 | Check the tightness of foundation bolts and side supports for crackness, Check tightness of cubical doors safety lock |  |  |  |
| 3 | Clean all dirt and dust deposit. Cleaning is by means of blowing out bushing off with a non- metallic brush or by rubbing with a cloth |  |  |  |
| 4 | Check the tightness of siecem coupler and earthing point |  |  |  |
| 5 | Check all wirings are secured and insulations are not damaged, burnt or eroded. Visually inspect all components for physical damage. |  |  |  |
| 6 | Auxiliary rectifier & Inverter (CG Module &WRE module): check the security of bolted terminals and mechanical mounting of larger components. |  |  |  |
| 7 | (IC Sch) Check the contacts of all modules and ensure proper tightness |  |  |  |

| क्र.सं SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) ###### Detail of Work/ Inspection (schedule) | की गयी कार्यवाही Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर/ टिप्पणी Sign/ Remarks |
| --- | --- | --- | --- | --- |
| C | # TRACTION CONVERTER (SR) |  |  |  |
| 1 | Examine all electrical equipment of traction converter for signs of dirt, corrosion, damage etc. Remove all dust/dirt deposits from the connection insulators. Check the tightness of foundation supports and side supports for crackness. Check tightness of cubical doors safety locks |  |  |  |
| 2 | # Check the main contactor, pre-charging contactor foundation bolt and tips for looseness and overheating by removing archutes. Check and ensure tightness of pre-charging contactor armature locking nut and strips |  |  |  |
| 3 | Check all connected fast on lug, bus bar connection, CTs, all LEM sensor connections, gate unit connections, pre-charging, MUB and its earth fault resistance, DC link and series resonance capacitor for leakage, tightness and overheating |  |  |  |
| 4 | Check the connection of earth fault relay. DC link sensor and all the earthing point of SR1, SR2 |  |  |  |
| 5 | Check BDV of traction converter oil (in GTO based loco), should be 30KV |  |  |  |
| 6 | Carry out DGA of traction converter oil by C&M Lab (in GTO base loco) |  |  |  |
| 7 | Check the oil level indicator situated on the conservator (Expansion Tank). If the oil is below the minimum mark-top up with the specified oil. Check for any signs of leakage, Range Oil level middle strip +/- 1.5” |  |  |  |
| 8 | Check for any leakage, loose, missing of screw from oil pipe joint, two flange joints, wire braided pipe, valve set vent plug, stoochi coupling, top bottom oil pipe, back body of valve set |  |  |  |
| 9 | SR-1 & SR-2 terminal, if difference of inductance between any two phase is < 0.005mH, allow the motor for next schedule |  |  |  |
| 10 | If the value is more than 0.005 mH, measure the inductance of individual TM and if difference between two phase of motor is < 0.015mH, allow the motor for next schedule. If > 0.015mH, remove the motor |  |  |  |
| 11 | Check the cable connection of Sichem connectors of differential amplifier provided below SR Oil pump. |  |  |  |
| 12 | Check ventilation fan of Traction converter. |  |  |  |

Remarks if any:

पर्यवेक्षक के हस्ताक्षर Signature of JE/SSE