LOCOMOTIVE SHED, SABARMATI

ELECTRICAL

[3-फेज विद्युत लोकोमोटिव आईए/आईबी/आईसी अनुसूची [निरीक्षण]

IA/IB/IC Schedule of 3-Phase Electric Locomotive [Inspection]

लोको संख्या Loco No. 	                    शिड्यूल Sch.                     शिड्यूल में लेने का दिनांक Date taken under Sch.

| ट्रैक्शन मोटर शिड्यूल TRACTION MOTOR SCHEDULE |  |  |  |  |
| --- | --- | --- | --- | --- |
| (A) Sch. IA/IB/IC: TM Schedule |  |  |  |  |
| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) ###### Detail of Work/ Inspection (schedule) | की गयी कार्यवाही Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर / टिप्पणी Sign/ Remarks |
| 1 | Examine all traction motors for signs of damage, dents or other defects caused by ballast. Check air outlets are not obstructed in any way. |  |  |  |
| 2 | Check traction power cables, speed sensor and temperature sensor cables and ensure that they are not chamfered or damaged in any way. |  |  |  |
| 3 | Check Tightness of both end cover bolts of Traction Motor. |  |  |  |
| 4 | Check the intactness of the junction box cover and bolt |  |  |  |
| 5 | Open TM Junction box at body side and check the tightness of connections |  |  |  |
| 6 | Check the condition of bellows and replace if required. |  |  |  |
| (B) Sch. IC: TM Schedule & Greasing |  |  |  |  |
| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) ###### Detail of Work/ Inspection (schedule) | की गयी कार्यवाही Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर / टिप्पणी Sign/ Remarks |
| 1 | Check for intactness of Grease nipple on NDE & DE side. Lubricate the both end bearings using a grease gun with specified grease and send old grease sample to Lab for testing metal content. |  |  |  |
| 2 | Check the condition of bearing by seeing the grease, which comes out from grease inlet. Check the condition of grease too. |  |  |  |
| 3 | Send grease sample to Lab for testing metal content, whenever grease available. |  |  |  |
| 4 | Check under truck TM cables for any rubbing against metallic body and secure cables properly. |  |  |  |
| 5 | If applicable following to be check: |  |  |  |
|  | i) Ensure tightness of all bolts of all Hurth coupling. |  |  |  |
|  | ii) Check oil level of Hutch coupling and send sample of oil to Laboratory. |  |  |  |
|  | iii) Check all TM coupling membrane for crack and oil leakages. |  |  |  |
|  | iv) Clean sleeve and star of Hurth coupling. |  |  |  |
|  | v) Check and replace bolt if found defective treads of bolts of sleeve and star of Hutch coupling. |  |  |  |
|  | vi) Check free movement of Hutch coupling. |  |  |  |

Remarks if any:

पर्यवेक्षक के हस्ताक्षर Signature of JE/SSE