SABARMATI DIESELS, LOCO CARE CENTRE, SABARMATI

Mechanical

IA/IB/IC Schedule of 3Phase Electric Locomotive [Inspection]

लोको  संख्या Loco No  	                  क्लास Class                  शिड्यूलमें लेने का दिनांक Date taken under Sch

|  | आगमनIncoming / शिड्यूलSchedule |  | अंतिमनिरीक्षणFinal Inspection |  |
| --- | --- | --- | --- | --- |
|  | # अपर डैक Upper Deck | अंडरट्रकUnder Truck | # अपर डैक Upper Deck | अंडर ट्रक Under Truck |
| Date/ Shift |  |  |  |  |
| Name of Technician |  |  |  |  |
| Designation/ Token No. |  |  |  |  |
| Signature |  |  |  |  |

| Customer Feed Back/Bookings (Record feedback given by Loco pilot) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sr. No. | Customer Feed Back/Bookings | Action Taken | Name of TCN / Attended By | Remarks, if any & Sign. of JE/SSE |

Inspection Wing Mechanical: Check and record Following

|  | CPA Loading Time (Pressure 0 to 8.0 kg/cm2) | Safety Valve Blowing | Pantograph-1 |  | Pantograph-2 |  | Compressor Loading Time (Pressure 8 to 9.5 kg/cm2) |  | CP Safety Valve Blowing | Unloaded valve |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Std. Value |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Raising | Lowering | Raising | Lowering | Comp-1 | Comp-2 |  |  |
|  |  | 8.5+ 0.25 kg/cm2 | 6 to 10 Second | Less than 15 Second | 6 to 10 Second | Less than 15 Second | 120 Second | 120 Second | 11.5+ 0.35 kg/cm2 | Operation time Approx. 12 Sec. |
| GI |  |  |  |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |  |  |  |

|  | MR safety Valve pressure |  | Duplex check valve setting/Brake Pipe Pressure by Auto Brake (A9) |  | Feed Pipe Pressure |  | Brake Cylinder Pressure by Direct Brake (SA9) |  | Horn Sound |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Blowing | Closing |  |  |  |  |  |  | Std. - Good |  |  |  |
| Std. Value | 10.5 0.2 kg/cm2 | 9.5–9.8 kg/cm2 | 5.0 0.1 kg/cm2 |  | 6.0 0.2 kg/cm2 |  | 3.50.1 kg/cm2 |  | Cab-1 |  | Cab-2 |  |
|  |  |  | Cab-1 | Cab-2 | Cab-1 | Cab-2 | Cab-1 | Cab-2 | LP | ALP | LP | ALP |
| GI |  |  |  |  |  |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Computerized Control Brake (CCB) System if applicable |  | Air Flow | Air Dryer |  | ADC | MRC Time By both CP 0-10kg/cm2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Std. Val. |  | Alerter and Penalty | Working | Working, Tower to change every minute (FTIL, SIL) & In every two minutes (KBIL) | Humidity Indicator Colour Std-Blue | Working | 3.5 Minutes (Max.) For 1750 LPM or [8.5 Minute for each 1450 LPM CP] |
|  | Passed successfully | Working |  |  |  |  |  |
| GI |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |

|  | ###### CPA Pressure build up time 8.5 kg/cm2 | ###### BP drop in 5 Minute | ###### FP drop in 5 Minute | ###### MR drop in 15 Minute | ###### Press BPPB to apply or release parking brake. Check Pressure in Parking Brake gauge |  | ###### VCB Pressure switch setting |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Std. Value | ###### 60 Sec. Max. | Max. drop 0.15 kg/cm2 | Max. drop 0.70 kg/cm2 | Max. drop 1.0 kg/cm2 | ###### 6.0+ 0.15 kg/cm2 |  | ###### Opens 4.5+ 0.15 kg/cm2 , Closes 5.5+ 0.15 kg/cm2 |
|  |  |  |  |  | ###### Cab-1 | ###### Cab-2 |  |
| GI |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |

|  | ACP on 4mm test plate |  | Auto brake controller Position | BP (kg/cm2) |  |  | BC (kg/cm2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cab-1 | Cab-2 |  | Cab-1 | Cab-2 |
| #### Std Values | Working |  | Run | 5.0 + 0.10 |  |  | 0.0 |  |  |
|  |  |  | Initial Application | 4.6 + 0.10 |  |  | 0.40 + 0.10 |  |  |
|  | Cab-1 | Cab-2 | Full Service | 3.35 + 0.20 |  |  | 2.50 + 0.10 |  |  |
|  |  |  | Emergency | Below 0.30 |  |  | 2.50 + 0.10 |  |  |
| Gen. Insp. |  |  |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |  |  |

|  | Loco pilot cabin condition |  | Wiper Operation |  | Look out Glass |  | Fire Extinguisher | Sand box | Sander Operation |  | Cleaning of Loco |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cab -1 | Cab-2 | Cab -1 | Cab-2 | Cab -1 | Cab-2 |  |  | Cab-1 | Cab-2 |  |
| Std. Values | Good | Good | Working | Working | Clear | Clear | Sealed | Full | Working |  | Cleaned |
| Gen. Insp. |  |  |  |  |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |  |  |  |  |

| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) ###### Detail of Work/ Inspection (Schedule) | कार्यवाही की गयी Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर/ टिप्पणी Sign/ Remarks |
| --- | --- | --- | --- | --- |
| (A) Pneumatic System (When Loco is energised check, attend and record) |  |  |  |  |
| 1 | Check correctness of reading of BP, FP, MR, BC, AFI & RS gauges. |  |  |  |
| 2 | Check on dummy of BP/FP pipe. |  |  |  |
| 3 | Check the working of PTDC |  |  |  |
| 4 | Check the proper functioning of all the gauges of the driver desk BP: (i) Normal (ii) with 7.5 mm hole BP should not drop below 4 kg/cm2 in 60 second. FP: (i) Normal (ii) with 5.5 mm hole FP should not drop below 4.5 kg/cm2 in 60 second. MR: (i) Mainline MR leakage without application of SA-9 Direct Brake (ii) Mainline MR leakage with application of SA-9 Direct Brake |  |  |  |
| 5 | (RS drop) Panto line air leakage 0.70 kg/cm2 in 5 minutes. |  |  |  |
| 6 | Check working of Panto selection switch. |  |  |  |
| 7 | Position of isolating cock on pneumatic panel. (70,74,136 in open condition & 47 in close condition.) |  |  |  |
| 8 | Drain moisture from MR and Inter Cooler. |  |  |  |
| 9 | Check functioning of air dryer for purging, cycle time and change over with MR pressure of 8.0 kg/cm2. |  |  |  |
| 10 | Check Position of the Air dryer cock (13,14 in service & 187 in isolated) |  |  |  |
| 11 | Check air dryer charging when compressor is in working. (FTIL -120 sec/SIL-180 sec) |  |  |  |
| 12 | ###### Check and attend leakage of air dryer, compressor valves, Pneumatic valves, Pneumatic panels & pipelines. |  |  |  |

| क्र.सं. SN |  | ###### कार्यवाही कार्य // निरीक्षण का विवरण(शिड्यूल) ###### Detail of Work/ Inspection (Schedule) | कार्यवाही की गयी Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर/ टिप्पणी Sign/ Remarks |
| --- | --- | --- | --- | --- | --- |
| B |  | BRAKE CONTROLLER (AUTO AND DIRECT) |  |  |  |
| 1 |  | Check the functioning of driver’s direct air brake & automatic train brake. |  |  |  |
| 2 |  | Check the free movement of the brake controller. |  |  |  |
| 3 |  | Perform repeated application and release of brakes with both auto and direct brake controller from both cabs and ensure that BP & BC pressure build up & drop is proper on the gauge. Ensure by physically going to individual main and parking brake cylinder that their operation is smooth and without air leakage. |  |  |  |
| C | LOOKOUT GLASS & SIDE WINDOWS |  |  |  |  |
| 1 | Clean any built up dirt and debris from the channel below the cab-sliding window. Clear the sliding window channel drains hole in the door opening. |  |  |  |  |
| 2 | Clean look out glass with detergent/water solution. |  |  |  |  |
| D | WASHER / WIPER |  |  |  |  |
| 1 | Check the operation of windscreen wipers using the manual handles. Rectify any fault found. |  |  |  |  |
| 2 | Check the condition of windscreen wiper blades. Replace if required. |  |  |  |  |
| 4 | Check the water level in the reservoir at No.1 and No.-2 end. Top up if required. Check wipers for proper operation |  |  |  |  |
| 6 | Check for damage of air and water pipes |  |  |  |  |
| 7 | Examine all components for wear, looseness scoring and cracks, renew any defective components. |  |  |  |  |
| 8 | Check seat-mounting points for integrity and ensure fixing or fully tightened. |  |  |  |  |
| J | Key interlocking: Check all keys are present & the complete system is operational |  |  |  |  |
|  | (IC Sch) |  |  |  |  |
| 9 | Check the wiper and parallel arm assembly for wear and damage. Replace the arm or any part if there is excessive wear or damage. |  |  |  |  |
| 10 | Clean the washer nozzle jet. (IC Sch) |  |  |  |  |
| E | Fire Extinguisher |  |  |  |  |
| 1 | Ensure availability of fire extinguishers and ensure that it is not expired. |  |  |  |  |
| 2 | Check the condition of CO2 gas cylinder, its regulator and pressure gauge in both cabs. Check the CO2 gas pressures. Refill if necessary |  |  |  |  |
| 3 | Clean pipeline by compressed air and check that there should not be any leakage. |  |  |  |  |
| 4 | CO2 Gas pressure and date: Cab-1 Cab-2 |  |  |  |  |
| 5 | Clean fire detection unit pipeline air intake holes with soft non- metallic brush & vacuum cleaner. |  |  |  |  |
| F | OIL LEAKAGE IN MACHINE ROOM / CLEANING OF MACHINE ROOM. |  |  |  |  |
| 1 | Visually inspect the machine room for any signs oil leakage from traction converter, transformer, D.C. Link, and series resonant circuit capacitors and other electrical equipment and carefully clean the same. (Caution : Electric supply room) |  |  |  |  |
| 2 | Clean the complete machine room with the air pressurized supply and vacuum cleaner to remove all the dust particles. |  |  |  |  |
| G | SAND GEAR SYSTEM |  |  |  |  |
|  | a) Check Sand box, cover gasket, Ejector and replace gasket if necessary. |  |  |  |  |
|  | b) Check all sander pipe for alignment and check fasteners also. |  |  |  |  |
|  | c) Check and ensure tightness of Sand box foundation bolt. |  |  |  |  |
|  | d) Ensure sander pipe height 60mm above rail level. |  |  |  |  |
|  | e) Refill sand in all sand boxes. |  |  |  |  |

Check and put () mark on the check box (CB) against each of the following items if the same has been checked. Write defects noticed and action taken in the Non-Schedule work table.

| UPPER DECK [Inspect, Attend and Record] |  |  |  |
| --- | --- | --- | --- |
| SN | Description | CB | Remarks |
|  | Loose, missing and broken components/parts. |  |  |
|  | Clamping and condition of its welding/fastener. |  |  |
|  | Condition and working of all pressure gauges. |  |  |
|  | Working of all brake valves. |  |  |
|  | Leakage of air (MR) |  |  |
|  | Gauges, securing of loco pilot cabin. |  |  |
|  | Operation of Wipers. |  |  |
|  | Co-Action working of A9 Auto brake. |  |  |
|  | Fire Extinguishers (prescribed Nos.) |  |  |
|  | Check tightness of compressor foundation bolt & abnormal sound. |  |  |
|  | check slackness of compressor fan and no vibration in comp. |  |  |
|  | Check tightness of air dryer foundation bolt. |  |  |
|  | Ensure Air dryer indicator blue colour |  |  |
|  | Ensure clamping of all pipe to avoid vibration and rubbing. |  |  |
|  | Ensure fitment of all split pins |  |  |
|  | Check tightness of all bolts of Loco Roof |  |  |
|  | Check window of both cab, shutter window door & corridor doors |  |  |

Check and put () mark on the check box (CB) against each of the following items if the same has been checked. Write defects noticed and action taken in the Non-Schedule work table.

UNDER TRUCK [Inspect, Attend and Record]

| SN | Description | CB | Remarks |
| --- | --- | --- | --- |
|  | Loose, missing and broken components/parts. |  |  |
|  | Clamping and condition of its welding/fastener. |  |  |
|  | Air leakage in bogie. |  |  |
|  | Overheated parts / Abnormal wear in parts. |  |  |
|  | Check and Ensure Brake block and Wheel clearance >10mm |  |  |

(Remarks if any & sign):

Signature and Name of Technician for Inspection Incoming with date & shift:

Signature and Name of JE/SSE for Inspection Incoming with date & shift:

Signature and Name of Technician for Inspection Final with date & shift:

Signature and Name of JE/SSE for Inspection Final with date & shift:

अन्य किसी भी जानकारी Any Other Information:

Signature of QAG SSE Incharge (Inspection)