SABARMATI DIESELS, LOCO CARE CENTRE, SABARMATI

IA/IB/IC SCHEDULE OF 3 PHASE ELECTRIC LOCOMOTIVE [UNDERFRAME]

| लोको संख्या Loco No |  | क्लास Class |  | शिड्यूल में लेने का दिनांक Date taken under Sch. |  |
| --- | --- | --- | --- | --- | --- |

UNDERFRAME IA/IB/IC SCHEDULE & RECORDS

| A-Check and record Hoot Wear, Flange Wear & Tread Wear (Record Wheel Diameter in Schedule or After TT) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wheel No | Wheel Tread dia | Root Wear | Flange Wear | Tread Wear | Wheel No | Wheel Tread dia | Root Wear | Flange Wear | Tread Wear |
| 1 |  |  |  |  | 7 |  |  |  |  |
| 2 |  |  |  |  | 8 |  |  |  |  |
| 3 |  |  |  |  | 9 |  |  |  |  |
| 4 |  |  |  |  | 10 |  |  |  |  |
| 5 |  |  |  |  | 11 |  |  |  |  |
| 6 |  |  |  |  | 12 |  |  |  |  |

B- Check and Record Vertical & lateral clearance (Primary & Secondary) as per TC/0082 (REV-1) dated 17/09/2015

| Standard Value | Primary vertical clearance (Cab-1 &2 Bogie) Actual Value Axle box & Bogie frame |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 27 to 35 mm |  |  |  |  |  |  |  |  |  |  |  |  |

| Standard Value | Primary lateral clearance (Cab-1&2 Bogie) Actual Value Axle box & Bogie frame |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 15 to 22 mm |  |  |  |  |  |  |  |  |  |  |  |  |

| Standard Value | Secondary Vertical clearance (Cab-1 Bogie) Actual Value Bogie frame & under frame |  |  |  |
| --- | --- | --- | --- | --- |
|  | CAB-1 LP Side | CAB-1 ALP Side | CAB-2 LP Side | CAB - 2 ALP Side |
| 32 to 60 mm |  |  |  |  |

| Standard Value | Secondary lateral clearance (Cab-1 Bogie) Actual Value Bogie frame & under frame |  |  |  |
| --- | --- | --- | --- | --- |
|  | CAB-1 LP Side | CAB-1 ALP Side | CAB-2 LP Side | CAB - 2 ALP Side |
| 45 to 55 mm |  |  |  |  |

C- Check and record wheel gauge below Parameters: (IC Schedule) WHEEL GAUGE [Permissible limit 1596+3-0.5  mm, (1596±0.5 mm for RB/New loco)]

| Axle No. | A | B | C | Average | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

Check and record below Parameters in Schedule or if TT is done:

| Axle No. | Standard (mm) |  | Cab-1 |  | Cab-2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location | Max | Min | RI | LI | R2 | L2 |
| Buffer Length | 635 | 615 |  |  |  |  |
| Buffer Height | 1105 | 1030 |  |  |  |  |
| Rail guard height | 118 | 115 |  |  |  |  |
| CBC Height | 1105 | 1030 |  |  |  |  |

|  |  | Cab-1 | Cab-2 |
| --- | --- | --- | --- |
| STRIKER WEAR PLATE | New 8 mm, Condemn 2 mm |  |  |
| SHANK WEAR PLATE | New 6 mm, Condemn 1 mm |  |  |

E- Measure D SHAKLE and write Coupling No.

CAB-1 							CAB-2

Check and Record Gear case Oil Level:

| TM | OIL FOUND | OIL TOP UP | TM | OIL FOUND | OIL TOP UP |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | 4 |  |  |
| 2 |  |  | 5 |  |  |
| 3 |  |  | 6 |  |  |

|  | Hand Brake Operation | CB Coupler Operation & Condition |  | Transition Screw Coupling Operation & Condition |  | CBC Lock Pin |  | HP/FF Pipe |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Std. Value | Working | Normal |  | Normal |  | Intact |  | Intact |  |
|  |  | Cab-1 | Cab-2 | Cab-1 | Cab-2 | Cab-1 | Cab-2 | Cab-1 | Cab-2 |
| Gen. Insp. |  |  |  |  |  |  |  |  |  |
| Final Insp. |  |  |  |  |  |  |  |  |  |

| Brake Piston Travel : |  |  |  |  |  | Brake Shoe release | Standard 3/9/10 mm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class of Loco | Standard value | Observed (mm) |  |  |  |  |  |
| WAG7/WAP7 | 107 to 117 mm | Location- | 1 | 2 | 3 | 4 |  |
| WAG9 |  | Location- |  |  |  |  |  |

| Sr. No. | Detail of Work/ Inspection | Name Technician | Sign/Remark |
| --- | --- | --- | --- |
| F | DRAFT GEAR AND COUPLERS |  |  |
|  | - Liberally apply general-purpose grease to the face of the center buffer coupler at the end of the Locomotive |  |  |
|  | - Check the intactness of the CBC bottom support plate bolts |  |  |
|  | - Check intactness and functioning of CBC operating handle. |  |  |
|  | - Check visually the striker casting of CBC assembly. |  |  |
|  | - Check for working of CBC, Lock and unlock it, locking arrangement, working of TC |  |  |
|  | - Check washer lock at knuckle pin bottom |  |  |
|  | - Check gap between Striker and Draft gear for draft gear slackness |  |  |
| G | BUFFERS |  |  |
|  | - Check buffers for any crack visually of the flange and intactness of foundation bolts, check nuts and split pins (4 nos. each), |  |  |
|  | - Liberally apply general purpose grease to the face of the buffers at the ends of the Locomotive. Also smear grease on the buffer head plunger. |  |  |
| H | SUSPENSION |  |  |
|  | - Examine the primary and secondary springs for signs of damage, cracks and broken end visually with torch light and tapping by a hammer & replace, if necessary |  |  |
|  | - Examine different types of dampers for oil leakage and intactness of fasteners and end fittings |  |  |
|  | - Check the condition of various Spheris block in the primary and secondary suspension arrangement and of various equipment mounted in the under frame like axle guide link, traction motor and its support arm, gear case support arm for their healthiness (it should not be crack) and replace, if necessary. |  |  |
| I | TRACTION BAR |  |  |
|  | - Check the well being and presence of the safety sling |  |  |
|  | - Check the push/pull rod pivot heads for damage |  |  |
|  | - Examine the traction link assembly for signs of damage. Any defective item should be renewed |  |  |
|  | - Check the fastener at the traction ends and those retaining the traction rod for security. If a fastener is loose, it must be renewed and tightened to the specified torque. Ensure intactness of the locking plate. SM1-241 |  |  |
|  | - RDPT of traction bar at end (flange) portion & V-Ring |  |  |
|  | - Housing flange. |  |  |
|  | - Examine limit chain and links for wear, corrosion or damage |  |  |
|  | - Inspect the safety slings, pins and R clips for wear and damage, |  |  |
|  | - Check Aclathan/Vulkollon ring against any crack or shifting |  |  |
|  | - Check gap in housing flange and traction link flange |  |  |
|  | - Torque modified bolt M20 X65 by 260Nm (SMI 241) |  |  |
| J | BOGIE FRAME AND BRAKE GEAR |  |  |
| i | BRAKE BLOCK |  |  |
|  | - Inspect the brake blocks for wear. If it is below the minimum limit, replace the complete locomotive set brake block. |  |  |
|  | - Check the brake block key security |  |  |
|  | - Check the brake rigging for loose, damaged or worn out parts Rectify as required. |  |  |
|  | - Examine the bogie frame, transom beam & damper base for signs of crack/damage |  |  |
|  | - Check fitment of Palm Pull rod J Bracket, hanger lever and TM safety rope wire. |  |  |
|  | - Inspect the rubber bumps for damage deterioration. Replace if required |  |  |
|  | - Examine all bogies mounted equipment (Including earthing straps) for signs of damage and security of fastening |  |  |
|  | - Inspect the brake rigging rotation points and component contact surfaces. Rectify any faults found. |  |  |
|  | - Check the earthing shunts provided on bogie and body. |  |  |
|  | Check intactness or axle box cover bolts and sealing. |  |  |
|  | - Inspect brake gear for any worn out loose, damaged, hanging or dragging parts. Confirm intactness |  |  |
|  | - Test for correct operation of the brake cylinder actuator unit. |  |  |
|  | - Check the proper tightness of parking brake & service brake pneumatic pipes. |  |  |
|  | - Check the external hitting marks & physical damage on the brake cylinder actuator unit. |  |  |
|  | - Check the condition of parking brake cylinder bellows If damaged, replace them. |  |  |
|  | - On parking brake cylinder pulling spindle & ring should be intact. |  |  |
|  | - Check proper functioning of parking brake & service brake cylinder. |  |  |
|  | - Check the operation (manual releasing) of the parking brakes |  |  |
|  | - Check the bogie isolating cock, should be in proper position. |  |  |
|  | - Check the wear of scrubber blocks, replace if required (WAP5only) |  |  |
|  | - Maintenance of Traction Motor support plate and bogie nose to Prevent crack/breakage of Traction Motor support plate (Holder for Traction motor suspension) should be done as per RDSO's SMI no. RDSO/2011/EL/SMI/0269 (Rev. '0') dtd, 18.05.11 |  |  |
| ii | WHEEL SET AND AXLE BOX |  |  |
|  | - Examine all the wheels visually for following defects. |  |  |
|  | a) Cracks: if any, requires replacement. |  |  |
|  | b) Flats: Length 40mm or more requires re profiling. |  |  |
|  | c) Cavities: Isolated cavities of length 15mm or more or multiple cavities of length 10mm or more and less than 50mm apart require re-profile/replacement. |  |  |
|  | d) Metal builds up Height 1mm, or above and length 50mm or more requires re-profiling. For length less than 50mm., remove the build up with hand tools. |  |  |
|  | e) Scoring and Grooving: Circumferentially around the wheel tread due to action of brake blocks require re-profile/replacement. |  |  |
|  | - Grease Axle box after 300000 KM Engine Run |  |  |
|  | - Check the temperature of axle boxes. The axle boxes which are running warm or reported to have higher temperature in comparison with others shall be attended on priority by physically opening of axle box cover and outer angle ring. The suitable maintenance shall be done either in-situ or by dismantling the axle box. |  |  |
| iii | GEAR CASE (WAP7/WAG9/WAG9H) |  |  |
|  | - Externally clean the gear case & its gauge glass. Inspect for any signs of oil leakage and any external hits/ damage. Verify integrity of gauge glass protective cover and clean oil sight glass for visibility. |  |  |
|  | - Check the oil level in the gauge glass and top up if required. Ensure intactness of the drain plug along with its sealing. Check its tightness. |  |  |
|  | - Check for any breakage of bolt and replace |  |  |
|  | - Check for intactness of all fasteners of gear case |  |  |
|  | - Tighten all gear case fasteners at prescribed torque |  |  |
|  | - Check the gear case oil visually and replace if found dirty or dust full level in the gear case and fill to the requirement |  |  |
|  | - Check Gear case oil for metal contamination |  |  |
|  | - Visually examine for fractures, the tyre profile for wear, flat, and skidding marks |  |  |
|  | - Check wheel root, flange and Tread wear & record Change all unserviceable Brake Blocks |  |  |
|  | - Check condition and operation of Hand Brakes. Lubricate & adjust as necessary. |  |  |
|  | - Adjust Brakes and lubricate union |  |  |
|  | - Lubricate Brake Gearing, Slack Adjusters, Buffers, CB Coupler Assembly |  |  |
|  | - Split the cotter pin, if new provided |  |  |

Check and put () mark on the check box (CB) against each of the following items if the same has been checked. Write defects noticed and action takes in the Non-Schedule work table.

UNDER TRUCK [Inspect, Attend and Record]

| SN | Description | CB No. | Remarks |
| --- | --- | --- | --- |
|  | Measure and record buffer & rail guard height. |  |  |
|  | If skidded, fill Wheel Sledding Complain Report Farm, FRMB/22 or if Biased wheel wear, fill up Check last for Based Wheel Wear Cause Invest, FILM/24) |  |  |
|  | Proper Fitment of Brake Block and as Key |  |  |
|  | Rubbing of brake block with wheel in brake release position |  |  |
|  | Movement of brake pull rod in brake applied position |  |  |
|  | Brake Shoe and Brake hanger Pin/Bolts |  |  |
|  | Brake piston movement. |  |  |
|  | Support arrangement for brake cross tie bars |  |  |
|  | CBC Drooping. |  |  |
|  | Operation of both Transition screw couplings |  |  |
|  | Operation of CBC, Operating Rod and Coupler lock Pin |  |  |
|  | Operation of CBC Knuckles |  |  |
|  | CBC retaining plate and support plate bolts |  |  |
|  | Side buffers and their mounting bolts. |  |  |
|  | Bolts of Cattle guards and back supports. |  |  |
|  | Fitment of Rail Guard |  |  |
|  | Gear case mounting bolts and ‘C’ clamp bolts |  |  |
|  | Suspension bearing cap bolts and it’s sealing. |  |  |
|  | Crack in TM Suspension taper roller bearing housing |  |  |

After TT Reading

| Wheel No | Wheel Tread dia | Root Wear | Flange Wear | Tread Wear | Wheel No | Wheel Tread dia | Root Wear | Flange Wear | Tread Wear |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 7 |  |  |  |  |
| 2 |  |  |  |  | 8 |  |  |  |  |
| 3 |  |  |  |  | 9 |  |  |  |  |
| 4 |  |  |  |  | 10 |  |  |  |  |
| 5 |  |  |  |  | 11 |  |  |  |  |
| 6 |  |  |  |  | 12 |  |  |  |  |

| Axle No. | Cab-1 |  | Cab-2 |  |
| --- | --- | --- | --- | --- |
| Location | RI | LI | R2 | L2 |
| Buffer Length |  |  |  |  |
| Buffer Height |  |  |  |  |
| Rail guard height |  |  |  |  |
| CBC Height |  |  |  |  |