SABARMATI DIESELS, LOCO CARE CENTRE, SABARMATI

IA/IB/IC SCHEDULE OF 3PHASE ELECTRIC LOCOMOTIVE

[COMPRESSOR &PNEUMATIC SYSTEM]

लोको संख्या Loco No  	                क्लास Class                   शिड्यूल में लेने का दिनांकDate taken under Sch

| क्र.सं. SN | ###### कार्यवाही कार्य // निरीक्षण का विवरण (शिड्यूल) ###### Detail of Work/ Inspection (schedule) |  | कार्यवाही की Action Taken | टीसीएन का नाम Name of TCN | हस्ताक्षर/ टिप्पणी Sign/ Remarks |
| --- | --- | --- | --- | --- | --- |
| A | ### मेन कंप्रेसर और बेबी कंप्रेसर Main Compressor and Baby Compressor |  |  |  |  |
| 1 | Check the pressure build up timing of each compressor independently in pre-inspection and take needful action during inspection. [ From 0 to 10 kg/cm2 i) With 1750LPM Comp-7 Minutes max. ii) With 1450LPM Comp- 8.5 Minutes max.] |  |  |  |  |
| 2 | Feel the vibrations of the compressor in pre-inspection and taken needful action during inspection. |  |  |  |  |
| 3 | Check for any external damage / abnormal sound / proper functioning of the compressor. |  |  |  |  |
| 4 | Check the complete compressor support brackets for their healthiness. |  |  |  |  |
| 5 | Visually examine the compressor mounting resilient blocks and take needful action. |  |  |  |  |
| 6 | Visually examine the various interconnecting pipelines and its support clamps of the compressors for their healthiness / air leakage. |  |  |  |  |
| 7 | Cleaning of input air filter and the primary oil filter. |  |  |  |  |
| 8 | Clean the breather assembly |  |  |  |  |
| 9 | Check and Drain the inter cooler / after cooler assembly and also observe its discharge and take needful action. |  |  |  |  |
| 10 | Ensure healthiness / intactness of various protection guards of compressor. |  |  |  |  |
| 11 | Check the condition of the coupling between compressor & motor |  |  |  |  |
| 12 | Visually check the compressor fan blade for their healthiness |  |  |  |  |
| 13 | Check the oil level in the compressor and top up as per requirement. |  |  |  |  |
| 14 | Carry out the inspection of each type of compressor in line with recommendation of respective manufacturer. |  |  |  |  |
| 15 | ### Check compressor foundation base for crack (For under slung compressor) |  |  |  |  |
| B | ### Aux. Compressor (CPA) |  |  |  |  |
| 1 | Clean the CPA externally. Clean Suction Air filter. |  |  |  |  |
| 2 | Check oil level and condition of oil and top up if required with the oil specified (Servo Press 150). |  |  |  |  |
| 3 | Check tightness of all nut bolts, foundation bolts. |  |  |  |  |
| 4 | Measure and record time & pressure of the auxiliary compressor |  |  |  |  |
| C | ### Sch IC: i) CP all discharge valves to remove. |  |  |  |  |
|  | ### ii) CP all discharge valves refit by overhauled |  |  |  |  |
|  | ### iii) CP oil to be change. |  |  |  |  |
|  | ### iv) Baby compressor/CPA oil to be change |  |  |  |  |
|  | ### CPA |  |  |  |  |
|  | ### CP-1 |  |  |  |  |
|  | ### CP-2 |  |  |  |  |
|  | ### v) Baby compressor/CPA safety valve (8.5 kg/cm2) to be replace by overhauled |  |  |  |  |
| D | ### AIR SUPPLY AND PNEUMATIC SYSTEM |  |  |  |  |
| 1 | ### NRV and unloader valves fail due to ingress of Carbon which extracted from CP. So, one cycle dismantling, checking and cleaning of NRV and unloader to be ensured in all type of 3-phase locomotives. |  |  |  |  |
| E | ### AIR DRYER |  |  |  |  |
| 1 | Inspect the air dryer for any damage and its mounting for any damage, crack in mounting plate, intactness of fasteners. Check for any hit mark on main reservoir and drain cock. Check for proper position of the air dryer angle cocks. |  |  |  |  |
| 2 | Check the integrity of the welding of the hanging support. |  |  |  |  |
| 3 | Check humidity indicators for blue color. White indications mean the air dryer is not operating effectively. Desiccant shall be changed, if needed. |  |  |  |  |
| 4 | Replace desiccant if found to be contaminated with either water or oil. |  |  |  |  |
| 5 | Testing of air dryer for proper functioning to be done as per Technical Circular no. RDSO/2011/EL/TC/0108, Rev.’0’ dtd. 7.3.11 for testing of air dryers and its efficacy on electric locomotives. |  |  |  |  |
| F | E-70 BRAKE SYSTEM (PNEUMATIC PANEL) |  |  |  |  |
| 1 | Visually inspect the pneumatic panel noting any sign of air leakages and rectify accordingly. |  |  |  |  |
| 2 | Clean the pneumatic panel externally. |  |  |  |  |
| 3 | Check for correct position of the cut out/isolating cock. |  |  |  |  |
| 4 | Check all the cable connections for looseness or deficiency of cleats. If any inspection section is to be informed (IC Sch) |  |  |  |  |
| 5 | Ensure provisioning and proper functioning of RS gauge, Switch, lamp & other gauges on pneumatic panel. |  |  |  |  |
| 6 | Check the sealing of connectors, flexible pipe, regulator, air tank etc. |  |  |  |  |
| G | BRAKE ISOLATING COCKS: Check for correct operations of BP & FP angle cocks. Check the condition of BP/FP pipe, cock & isolating handle for any hit mark or crack. Check correct condition of Bogie Isolating cock. |  |  |  |  |
| H | ### BRAKE HOSES: Examine the brake hoses at the ends of the locomotive to make sure that they are not chafed or damaged. Check coupling cocks for correct operation. |  |  |  |  |
| I | ### AIR RESERVOIR |  |  |  |  |
| 1 | Blow down and drain the reservoir until exhaust of complete water and oil. Check all drain cocks are closed on completion. |  |  |  |  |
| 2 | Check for any hit mark on main reservoir & drain cock |  |  |  |  |
| 3 | Ensure intactness / tightness of main reservoir foundation bolts. |  |  |  |  |
| 4 | Check the rubber gasket at the inspection cover of air reservoir for any damage/leakage. |  |  |  |  |
| 5 | Check the pressure regulator operates correctly |  |  |  |  |
| J | THROTTLE VALVE (IC Sch): Overhaul the throttle valve and change the rubber parts. |  |  |  |  |
| K | IN LINE FILTER (IC Sch): Remove the filter element and clean thoroughly. |  |  |  |  |
| L | HORN & HORN SWITCH |  |  |  |  |
| 1 | Check condition of horn handle for any damage. Check for proper operation of horn. |  |  |  |  |
| 2 | Check the condition of horn boots and replace if required |  |  |  |  |
| 3 | Remove the horn & horn switch and Overhaul. Replace the diaphragm of horn. (IC Sch) |  |  |  |  |
| 4 | Check all angle cocks for proper operation |  |  |  |  |
| 5 | Ensure proper colour code of all the angle cocks. |  |  |  |  |
| 6 | Check the tightness of the brake electronics control cards holding screws. |  |  |  |  |
| 7 | Check all the pipes for leakage or damage. Check all the clamps for any damage and looseness. |  |  |  |  |
| M | ### वायुवीय प्रणाली PNEUMATIC SYSTEM |  |  |  |  |
| 1 | ### MR Safety valve, ADC, Panto throttle valve (Only for conventional Pantograph) replace by Overhaul in IC Sch. |  |  |  |  |

Signature and Name of Technician  	  	     				Signature and Name of JE/SSE