SABARMATI DIESELS, LOCO CARE CENTRE, SABARMATI

CHECK SHEET FOR 3PHASE HR PANTOGRAPH ASSEMBLY (IA/IB/IC)

लोको संख्या Loco No  	                क्लास Class                       शिड्यूल में लेने का दिनांक Date taken under Sch

SCHEDULE: .............. PANTOGRAPH SR.NO. /Make//Type: .................... LOCATION: …………..

| SN | ###### Detail of Work/ Inspection (schedule) | Standard value | Action Taken |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PT-1 | PT-2 |  |
|  | Ensure that wearing strips are without sharp edges and no gap between end strip and wearing strip. | Smooth and no gap |  |  |  |
|  | Ensure availability of bolts, nuts, lock nuts, spring, washer, split pin etc. | All intact |  |  |  |
|  | Check the rubber air pipe for any leakage | No leakage |  |  |  |
|  | Check the rubber air bellow for any leakage/damage/rubbing | No leakage |  |  |  |
|  | Check lower frame assembly for any crack and play | No crack |  |  |  |
|  | Check upper frame assembly for any crack and breakage | No crack |  |  |  |
|  | Check coupling rod assembly for crack and its fitting intactness. | No crack & intact |  |  |  |
|  | Check diagonal rod for any crack both end side. (by DPT) | No crack |  |  |  |
|  | Check plain bearing eye bush for freeness. | free |  |  |  |
|  | Check ball bearing friction of Base frame, Upper frame and Coupling rod. | ok |  |  |  |
|  | Check Cam wear on top of cable rope guide surface. | OK |  |  |  |
|  | Check condition of all shunts (not more than 5% strand broken) | OK |  |  |  |
|  | Check pantograph pan for any external hitting marks | No hitting marks |  |  |  |
|  | Verify all welded joint for cracks. | OK |  |  |  |
|  | Check the control box for any air leakage | No air leakage |  |  |  |
|  | Check the condition and operation of hydraulic damper. | Normal |  |  |  |
|  | Check Parallel guide bar for free function of Panto Pan Head | working |  |  |  |
|  | Ensure that the thickness of wearing strips above the condemning size | New – 39 mm Condemn – 26 mm |  |  |  |
|  | Check the static contact force of pantograph at 6.4 kg/cm2 MR pressure | 70 N / 7Kg Load Test |  |  |  |
|  | Check the raising and lowering time of pantograph | 6 to 15 second |  |  |  |
|  | Check the horizontality of panto pan with sprit level. | Level OK |  |  |  |
|  | Check the function of auto drop device (ADD) | Working |  |  |  |

| SN | ###### Detail of Work/ Inspection (schedule) | Standard value | Action Taken |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | PT-1 | PT-2 |  |
|  | Check the function of over reach detector (ORD) | Working |  |  |  |
|  | Check air filter for air leakage or not draining. | No leakage |  |  |  |
|  | Check the all pneumatic valve (3/2 valve, throttle valve and safety valve) for any leakage and proper function. | No leakage |  |  |  |
|  | Check pressure regulator for any leakage and proper function. | No leakage |  |  |  |
|  | Check the tightness of all bolts and thread locking compound to be provided in bearing base bolts. | Provided |  |  |  |
|  | Examine the roofline conductors for defects/looseness at joints. | No defect |  |  |  |
|  | Check the roof for any foreign material such as wire / cloth pieces. | No foreign material |  |  |  |
|  | Check tightness, cleanliness and wear-tear of shunt connection between Panto and Roof line. | Tight, clean |  |  |  |
|  | Check that the pan head locking pins springs are in place and secure. Check that the head moves freely on its mounting pins attached to the apex frame. If movement is not free, check that the head is not twisted, the side beam are not bent/twisted & mounting pins are not loose or bent. | OK |  |  |  |
|  | Check tightness of the flexible shunt connecting screws. When a loose screw is found the shunt should be disconnected from the head &/or the pantograph frame, the mating surface cleaned & the shunt reconnected with new screws. Check all shunts for frayed strands and renew any shunt in which 5% or more of the strands are frayed. | OK |  |  |  |
|  | IC Sch: Clean filter element, if found brownish then replace with new. | Clear |  |  |  |

Remarks if any:

Signature and Name of Technician         						Signature and Name of JE/SSE