SABARMATI DIESELS, LOCO CARE CENTRE, SABARMATI

CHECK SHEET FOR 3PHASE Carbody and OCB (IA/IB/IC)

लोको संख्या Loco No  	            क्लास Class              शिड्यूल में लेने का दिनांक Date taken under Sch

| SN | ###### Detail of Work/ Inspection (schedule) | Action Taken | Remarks |
| --- | --- | --- | --- |
| A | CARBODY FILTER: Check condition and clean car body filter and refit. |  |  |
| 1 | Thoroughly clean all roof and side body filters with water jet. |  |  |
| 2 | Check tightness of Roof joint clamp and Roof top bolt. |  |  |
| 3 | Remove Louvers& Fly screen and clean it, Repair if found defective. |  |  |
|  | IC sch. |  |  |
| 4 | Remove the tread plate and clean the drainage channels. Check and adjust, if necessary, the door-operating rod. |  |  |
| B | # OIL COOLING UNIT CASING WITH RADIATOR |  |  |
| 1 | Remove all dust, dirt and debris from the radiator chamber via the machine room access cover. (By using vacuum cleaner). |  |  |
| 2 | Visually check the conservator tank along with its gauge glass for any sign of crack / damage / oil leakage. |  |  |
| 3 | Measure air delivery before and after cleaning of radiator. |  |  |
| 4 | Measure vibrations of oil cooling casing using vibration analyzer and take appropriate action |  |  |
| 5 | Visually check the casing and support arrangement for oil pumps on the casing for any sign of crack of the damage |  |  |
| 6 | Visually check the oil cooler radiator for any oil leakage from VPTFP and VPSR and external damage from top to bottom. |  |  |
|  | IC Sch |  |  |
| 7 | a) Clean radiator with hot water jet and check that the radiator is not blocked. |  |  |
| 8 | b) Tighten the front foundation bolts of casing mounted on to the radiator and check intactness of jail and Z-clamp welding crack. |  |  |