SABARMATI DIESELS, LOCO CARE CENTRE, SABARMATI

IA/IB/IC Schedule of 3 Phase Electric Locomotive

लोको  संख्या  Loco No  	                  शिड्यूल में लेनेका दिनांक  Date taken under Schedule

| Sr.No | Details of Work / Inspection (Schedule) | Action Taken | Name of TCN | Sign / Remarks |
| --- | --- | --- | --- | --- |
| A | VCB |  |  |  |
| 1 | # Check insulator for crack, VCB roof bus bar, chips and flash marks. If damage in service, renew it |  |  |  |
| 2 | # Clean insulator, other parts that are dirty with soft, clean & dry cloth. (Do not use cleaning products based on fluoridated or chlorate compounds or sodium meta silicate), apply Silicon Spray. |  |  |  |
| 3 | Check contact spring (Earthing contact) |  |  |  |
| 4 | Check moving contact lever of auxiliary switch and tension spring & its mounting bolts for breakage |  |  |  |
| 5 | Check the cables & lugs for broken cables, lugs looseness |  |  |  |
| 6 | Check the main bushing condition, its connection and foundation bolts and clamps. |  |  |  |
| 7 | Check the tightness of HV Connections (Torque 70Nm) |  |  |  |
| 8 | IC Sch: a) Check for damage to connection of the earthing isolator, Cleaning & greasing, if necessary. |  |  |  |
|  | b) Check the torque of VCB fixing screw (Torque 70Nm) |  |  |  |
|  | c) Remove the bottom cover and check that the control air and electrical connection are tightened & Ensure that there is no leakage from pressure regulator |  |  |  |
| B | MAIN TRANSFORMER |  |  |  |
| 1 | Read off the oil level on the gauge situated on the conservator. Top up the oil as necessary and check for any signs of leakage. Clean the gauge with dry cloth |  |  |  |
| 2 | Inspect the colour of the silica gel. If it is pink, replace with blue silica gel |  |  |  |
| 3 | Examine the flanges of the pipe couplings and flexible hose that link the transformer and conservator |  |  |  |
| 4 | Check the availability of hosepipe over the oil pipe compensator and check stoochi coupling pipes for proper layout/fitment. |  |  |  |
| 5 | Check the main Transformer and its protection cover for any damage, crack & oil leakage. Also refer instructions contained in RDSO’s letter no. EL/3.2.1/3-Ph / TC-0076 for arresting oil leakages cases” |  |  |  |
| 6 | Check the deformity of Transformer drain cock protection cover guard if deformed replace it |  |  |  |
| 7 | Perform the sample test on transformer oil. Check specific value of BDV, DGA, moisture and acidity and condition monitoring of loco transformer by Dissolve Gas Analysis. (RDSO/SMI/138 & OEM Doc. Dt. 27th Nov, 1995) |  |  |  |
| 8 | Check visually the condition of transformer foundation bolt and Nylon lock nuts for proper locking. |  |  |  |
| 9 | Check the Ecable. Clean and visually inspect the insulator condition and Electrical connection of transformer and converter for crack & flashed mark and ensure red marking |  |  |  |
| 10 | Visually inspect for leakage, damage, burning/overheat signs and all fixing clamps condition of oil cooling metallic pipes, Prismatic level gauge, TFP bushing, HV bushing, Hose pipe etc. |  |  |  |

Signature and Name of JE/SSE

| Sr.No | Details of Work / Inspection (Schedule) |  |  |  | Action Taken | Name of TCN | Sign / Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C | # EARTHING CONNECTION. |  |  |  |  |  |  |
|  | - Check the earthing connection tightening torque 70Nm. |  |  |  |  |  |  |
|  | - Connection of earthing system in 3-phase locomotives type WAP5, WAP7 & WAG9 (As per RDSO/2007/EL/SMI/248 Rev.’0’ dated 22.11.2007) |  |  |  |  |  |  |
|  | - Check Earthing shunt of Bogie to Body |  |  |  |  |  |  |
|  | Axle box 1-3 |  | Axle box 2-4 |  |  |  |  |
|  | Axle box 5-7 |  | Axle box 6-8 |  |  |  |  |
|  | - Check Earthing shunt of Axle to Bogie |  |  |  |  |  |  |
|  | Axle box 2 |  | Axle box 3 |  |  |  |  |
|  | Axle box 6 |  | Axle box 7 |  |  |  |  |

Signature and Name of JE/SSE

| SN | Repairs | Action Taken | Attended By |
| --- | --- | --- | --- |